Prof. S. BARANOV

Introduction to Computer Engineering (83-252-02, 03)

Exam A

Instructions: 1. Duration for exam – 3 hours.

2. Materials for help – closed.

3. Each answer must be explained.

Problem 1 (30%). For given ASM G1:

- Construct Mealy FSM S1 (direct transition table);

- Construct inverse transition table for Mealy FSM S1;

- Construct combined FSM S2.

Problem 2 (20%).    Decompose given FSM S3 with the following partition at the set of states:

π = {A1, A2, A3, A4, A4, A4};

A1 = {a1};   A2 = {a2};   A3 = {a3};   A4 = {a4};   A5 = {a5};   A6 = {a6};

FSM S3:

a1   a2   x6          y8y9      1

a1   a5   ~x6*x7      y6        2

a1   a5   ~x6*~x7     y3y6y10   3

a2   a2   x4*x1       y1y2      4

a2   a6   x4*~x1      y3y4      5

a2   a4   ~x4         y4        6

a3   a6   1           y3y4      7

a4   a1   x5          --        8

a4   a2   ~x5*x1      y8y9      9

a4   a6   ~x5*~x1     y3y4      10

a5   a2   x1*x2*x3    y1y3      11

a5   a3   x1*x2*~x3   y6y7      12

a5   a2   x1*~x2      y1y3      13

a5   a4   ~x1         y4        14

a6   a5   x6          y6y7      15

a6   a6   ~x6         y3y4      16

Problem 3 (20%). Construct logic circuit for transitions into state a2, using factoring and delete equal gates. Use algorithm discussed in lectures (without Boolean expressions). Realize logic circuit on NOR/NAND gates.

a3   10   a2   00   x3*x4                y1y3     --     7

a3   10   a2   00   x3*~x4               y1y3     --     8

a1   01   a2   00   x1*x6*x2*x5*x3*x4    y1y3     --     9

a1   01   a2   00   x2*x5*x3*x4          y1y3     --     10

a2   00   a2   00   x2*x5*x3*~x4         y1y3     --     11

a1   01   a2   00   x1*x6*x2*x5*x3*~x4   y1y3     --     12

# Problem 4 (30%).     Construct logic circuit for the abstract automaton, using given encoding for inputs, outputs and states.

|  | u1 | u2 | -- | u3 | u1 |
| --- | --- | --- | --- | --- | --- |
|  | a1 | a2 | a3 | a4 | a5 |
| z1 | a2/w2 | a1/w1 | a3/- | a4/w2 | a3/w1 |
| z2 | a4/-- | -- | a2/w2 | -- | a2/w2 |
| z3 | a1/w1 | a5/w3 | -- | a5/w1 | a1/w1 |

|  | y1 | y2 |  |  | r1 | r2 |  |  | x1 | x2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w1 | 0 | 0 |  | u1 | 0 | 0 |  | z1 | 0 | 0 |
| w2 | 0 | 1 |  | u2 | 0 | 1 |  | z2 | 0 | 1 |
| w3 | 1 | 0 |  | u3 | 1 | 0 |  | z3 | 1 | 0 |

|  | t1 | t2 | t3 |
| --- | --- | --- | --- |
| a1 | 0 | 0 | 0 |
| a2 | 0 | 0 | 1 |
| a3 | 0 | 1 | 0 |
| a4 | 1 | 0 | 0 |
| a5 | 0 | 1 | 1 |