Prof. S. Baranov

Dr. R. Finkelshtein

Introduction to Computer Engineering (83-252-02, 03)

Exam C

Instructions: 1. Duration for exam – 3 hours.

2. Materials for help – closed.

Problem 1 (30%). For given ASM:

- Construct Mealy FSM S1 (direct transition table);

Problem 2 (20%).    Decompose given FSM S2 with the following partition on the set of states:

π = {A1, A2, A3, A4};

A1 = {a1,a4};   A2 = {a3,a6};   A3 = {a2 };   A4 = {a5, a7 };

FSM S3:

a1   a2   x1*x12*x11                y2      1

a1   a3   x1*x12*~x11*x8            y3y4    2

a1   a3   x1*x12*~x11*~x8*x5        y4y6    3

a1   a4   x1*x12*~x11*~x8*~x5*x6    y5      4

a1   a2   x1*x12*~x11*~x8*~x5*~x6   y2      5

a1   a7   x1*~x12*x11               y9y10   6

a1   a4   x1*~x12*~x11*x4*x2        y5      7

a1   a3   x1*~x12*~x11*x4*~x2       y3y4    8

a1   a3   x1*~x12*~x11*~x4*x2       y4y6    9

a1   a2   x1*~x12*~x11*~x4*~x2      y2      10

a1   a1   ~x1                       --      11

a2   a1   1                         y1      12

a3   a2   x3                        y2      13

a3   a3   ~x3                       --      14

a4   a5   x12                       y4y7    15

a4   a3   ~x12                      y4y6    16

a5   a2   x7                        y2      17

a5   a5   ~x7                       --      18

a6   a3   x9*x10                    y3y4    19

a6   a3   x9*~x10                   y4y6    20

a6   a2   ~x9                       y2      21

a7   a6   1                         y8      22

Problem 3 (20%). For the given logic circuit, construct logic circuit with NOR-NAND gates.

# Problem 4 (30%).   Construct two-level minimized logic circuit for the given abstract automaton, using optimal encoding for outputs and states. Use D flip-flops as memory elements. For minimization use Karnaugh maps at page 5.

|  | a1 | a2 |
| --- | --- | --- |
| z1 | a2/w4 | a1/w2 |
| z2 | -- | a2/w3 |
| z3 | a1/- | a2/w4 |
| z4 | a2/w1 | a1/w2 |
| z5 | -- | a1/w3 |
| z6 | a1/- | a2/w4 |
| z7 | a2/w4 | a1/w2 |
| z8 | -- | a1/w3 |
| z9 | a1/- | a2/w1 |

Good luck!

Prof. Samary Baranov