מעגלים ספרתיים אלקטרוניים - פתרון חלקי לתרגיל בית 1

שאלה 1

שאלה 2 – סעיף א'

המשוואות עבור תרגילים 1 ו-2:

שאלה 3

שאלה 4

Let’s denote the drain of M1 (which is also the source of M2) by X.

A. Given: M1 is cut  VGS1 < VT1  VG < VT1

VGS2 = VG – Vx < VG – 0 < VT1 = VT2  M2 is also cut, QED.

B. Assume M1 is saturated  VGS1 > VT1, VDG1 > -VT1 

Vx – VG > -VT1 VG-Vx < VT1  VGS2 < VT2  M2 is cut.

By the given assumption, M1 is also cut  contradiction to the assumption that M1 is saturated.

 M1 can’t be saturated, QED.

C.

I1/K1+I2/K2 = = 2I1/K = 2(VG-VT1)Vx-Vx^2 + 2(VG -Vx- VT2)(VD-Vx)-(VD-Vx)^2 =

[T1=T2 = T]

= 2VGVx – 2VxVT – Vx^2 +

2VGVD - 2VGVx - 2VDVx + 2Vx^2 - 2VDVT + 2VxVT - VD^2 - Vx^2 + 2VDVx

= 2VGVD - 2VDVT - VD^2 = 2(VG-VT)VD - VD^2 =

= I/Keq

Where we use one transistor with Keq connected to G, D and S.

 I/Keq = 2I/K1  Keq = K1/2. QED

The equal transitor is linear, because we know that M2 is linear, ie

VDG < - VT

At the tutorial: explain now why at linear opeartion Ron is a good approximation.

D. T1=T2 = T, I1=I2 

2(VG-VT)Vx-Vx^2 = (VG -Vx- VT)^2  … 

2Vx^2 – 4(VG-VT)Vx + (VG-VT)^2 = 0 

Vx = [1+1/sqrt(2)](VG-VT) or Vx = [1-1/sqrt(2)](VG-VT),

 (VG-Vx-VT)^2  = (VG-VT)^2/2

I = K/2(VG-VT)^2

The 2 transistors are equal to one transistor with Keq = K1/2, connected to G, D and S.

This transitor is saturated, because we know that M2 is saturated, ie

VDG > - VT

E. Given: VG <= VD  VD – VG >= 0 > -VT2  VDG > -VT1  M2 is always saturated (or cut), not linear.

F. We’ve just seen that when VG <= VD, M2 is always saturated (or cut).

M1 is saturated  Vx - VG > -VT1  VG < Vx+VT1 = Vx+2

I1 = I2 (VG-VT1)^2 = (VG-Vx-VT2)^2. As both terms within the parentheses are positive, we may take the sqrt:

VG-2 = VG-Vx-1 Vx=1v

We have just seen that VG < Vx+2  VG < 3v

On the other hand, conducting condition for M1 is: VGS > VT1  VG > 2v.

Finally: 2v < VG < 3v

G. I=I1+I2. Keq=K1+K2

שאלה 5

שאלה 6

שאלה 7

A.

Vgs1 = Vdd – Vo

Vdg1 = 0 > -Vt1  N1 is always cut or saturated (not linear).

Vgs0 = Vi  N0 conducts when Vi > 1.2v

Vdg0 =  Vo – Vi N0 is saturated when Vi > 1.2v AND Vo-Vi>-1.2v.

Voh = ?  Maximal Vout is received when Vgs1 = Vt1  Vo = Vdd – Vt1 = 8.8v

At that point, N1 is at the switching saturatation / cut. N0 is cut.

For all other points, N1 is saturated.

Vol = ? 

K0[2(Vi-Vt0)Vol – Vol^2] = K1(Vdd-Vo-Vt1)^2

Note: Vo=Vol, Vin(ol)=Voh

K0/K1[2*7.6Vol – Vol^2] = (8.8-Vo)^2  Vol=0.747v (the other option is not physibale).

Vm = ? 

K0(Vm – Vt0)^2 = K1(Vdd-Vm-Vt1)^2

K0/K1(Vm – 1.2)^2 = (8.8-Vm)^2

As both sides are positive, we may take out a square root. Denote: Kr=sqrt(K0/K1).

Kr(Vm-1.2)=8.8-Vm

Vm = (8.8 + 1.2Kr) / (Kr + 1) = 3.4v

Vih = ? 

K0[2(Vi – Vt0)Vo-Vo^2] = K1(Vdd-Vo-Vt1)^2

2ViVo – 2.4Vo –Vo^2 = K1/K0(8.8-Vo)^2

Vi = [2.4Vo + Vo^2 + K1/K0(8.8 – Vo)^2]/2Vo

Note: K1/K0 = 1/6

Vi = 1.2 + Vo/2 + 8.8^2/12Vo – 17.6/12 + Vo/12

Vi = 1.2 – 17.6/12 + (7/12)Vo + 8.8^2/12Vo

Derivate by vi 

1 = -7/12 + 6.45/Vo^2  Vo(ih) = 2.02v, Vih=4.11

Vil = ? 

K0(Vi – Vt0)^2 = K1(Vdd-Vo-Vt1)^2

Kr(Vi – 1.2) = 8.8-Vo

Derivate by vi 

Kr = 1  Wrong.  No Vil by derivative. Therefore, we shall choose Vil by the “knee” point of the slope of Vo=f(Vin)  1.2v

| Vol | Vih | Vm | Vil | Voh |  |
| --- | --- | --- | --- | --- | --- |
| L | L | S | S | C | מצב N0 |
| S | S | S | S | S / C | מצב N1 |
| 8.8 |  | 3.4 | 1.2 | < 1.2 | Vin [v] |
| 0.747 | 2.02 | 3.4 | 8.8 | 8.8 | Vout [v] |

B.

Pstat = [Poh + Pol]/2 = Pol/2

This Poh = 0, because when Vo=Voh N1 is cut  no current, so no waste of power.

Pol = Vcc*I = Vcc*K1(Vdd-Vo)^2 = 100/3(8.8-0.747)^2 = 2160 uW

 Pstat = 1,080mW

C.

Q=CV  I = Cdv/dt  dt = Cdv/I

Tphl = ?

Denote: Ve = (Vol+Voh)/2

An approximation: assume I is constant – the average of I at the beginning and at the end.

I = [Ioh + Ie]/2

Where Ie is the current when Vout = Ve.

dv = Voh – Ve = (Voh-Vol)/2 = 4.03v

Note:

- For getting Tphl in an inverter, we give HIGH voltage (Voh) at the input.

- As this is a discharge, the discharge current is I0 – I1.

Ioh = ?

Vgs0 = Voh,  Vdg0 = Voh-Voh = 0 > -Vt0  N0 is saturated.

When Vout = Voh, I1 is cut.

 Ioh = I0(oh) – I1(oh) = I0(oh) = K0(Voh-Vt0)^2

Ie = ?

Vgs0 = Voh,  Vdg0 = Ve-Voh <-Vt0  N0 is linear.

Vgs1 = Vcc – Ve > Vt1  N1 conducts, and is saturated.

 Iol = I0(ol) – I1(ol) = K0[2(Voh-Vt0)Ve – Ve^2] – K1(Vdd-Ve-Vt1)^2

 …  Tphl (approx.) = 3.67 ns.

Tphl (exact) = ?

Use an integral. We should divide the range to a few integrals, by the transistors’ modes. Luckily, N1 is always at the same mode – saturated (though sometimes saturated with current 0). So we have 2 integrals, by the different modes of N0 – sat / lin.

The breaking point between saturation and linearity at N0 is gotten when:

Vdg0 = Vo – Vin = Vo – Voh = -Vt0  Vo = Voh – Vt0 = 7.6v

Threrefore, first integral is from Voh to Voh-Vt0, using N0 saturation formula. Second integral is Voh-Vt0 to Ve, using N0 linear formula. At both, N1 is saturated.

Integrals are of Cdv / {I0(Vo) – I1(Vo)}.

E.

Body effect occurs only in N1, and is negligible when Vo is small, ie at Vol, Vih.

Let’s calculate Voh, considering the body effect:

Voh = Vcc – Vt1 = 8.8 – 0.5[sqrt(Voh+0.6) – sqrt(Voh)]

For solving this equation, we use the following trick: adding 0.6 to both sides. Then

Voh + 0.6 = 9.4 - 0.5[sqrt(Voh+0.6) – sqrt(0.6)]

Now, by denoting: X = sqrt(Voh + 0.6)  X^2 = 9.4 – 0.5[X – sqrt(0.6)]  … 

X = 3.38v or -2.88v  Voh’ = 10.82v or 7.7v.

The second answer is the feasible one, and we see it’s indeed significantly different then Voh without considering the body effect,

Note: there are cases when an iterative solution is used, ie:

- Assume Vt1 = … (slightly above Vt1,0).

- Calculate Vout.

- Calculate Vt1’ by this Vout. If Vt1’ != Vt1, define Vt1 = Vt1’ and go to b.

- Stop when the gap ABS|Vt1’ – Vtt1] are within the predefined range.

שאלה 8

A. Pseudo NMOS.

B. It’s possible to switch Q1, Q2 by an equal transistor (denoted Q12)

1/K1 + 1/K2 = 1/K12  K12 = 25uA/V^2

Voh = ?

For getting Voh, let’s input the lowest possible input. It’s enough to input Vin=Vtn=0.5v, and then Q12 is cut. Then

Vgs4 = 0 – Vdd = -Vdd < Vtp  Q4 always conducts – until Vout=Vdd.

Note that when Vout=Vdd, Q3 conducts, and then X (the source of Q2) is also Vdd.

Voh=Vdd=2.5v

Vol = ?

For getting Vout = Vol, let’s insert Vin=Voh=Vdd.

Then, Vgs12 = Vdd, Vds = Voh

Vdgn = Voh-Vdd, so probably Vdg<-Vtn  Q12 is linear.

Vdgp = Vol – 0 = Vol

Let’s assume Vol > 0.5v = -Vtp  Q4 is linear.

Note that Vout is rather low, therefore there’s only a little conductance in Q3.

In = Ip  K12[2(Vdd-Vtn)Vol – Vol^2] = K4[2(Vdd+Vtp)(Vdd-Vol) – (Vdd-Vol)^2]

 Vol = 0.21v < 0.5v  The assumption that Q4 is linear was wrong.

So, let’s assume Q4 is saturated:

In = Ip  K12[2(Vdd-Vtn)Vol – Vol^2] = K4(Vdd+Vtp) ^2  Vol=0.21v

C) When Vout = Voh  Pstat=0.

When Vout = Vol:

P = DELTA(V)*I = 2.5v*Ip = 2.5v * 5uA/v^2 * 4v^2 = 50uA*v = 50uW