מעגלים ספרתיים אלקטרוניים - פתרון חלקי לתרגיל בית 2

שאלה 2

First, let's understand the connectivity of M2.

Potential divider between Vout and Vdd:

VG2 = Vout + (Vout - Vdd) * R2 / (R1 + R2) = Vout + (Vdd-Vout)/3 =

= Vdd/3 + 2*Vout/3

VDG2 =  Vout – VG2 = Vout – Vdd/3 – 2*Vout/3 = (Vout – Vdd)/3

<= 0 < 0.8v = -VTP 

VDG2 <= -VTP  When M2 conducts it's saturated (NOT linear).

Now, let's find the required points.

Voh ? 

Conduction condition for M2: VGS2 < VTP 

VGS2 = VG2 – Vdd = Vdd/3 + 2*Vout/3 – Vdd = 2*(Vout – Vdd)/3 < VTP = -0.8

 Vout*2/3 – 10/3 < -0.8  2Vout -10 < -2.4

Vout < 3.8v  Voh = 3.8v

For finding Vol, we should define Vin = Voh = 3.8v

In that case, VDG1 = Vout – Vin = Vout – 3.8

As Vout is expected to be low at that case, let's assume:

VDG1 = Vout – 3.8 < -VT = -0.8  M1 is linear.

I. IDS1 = ISD2 + IR1

The values to assign in Eq. I. are:

VGS1 = Vin = Voh = 3.8v

VDS1 = Vout = Vol

IDS1 = Kn[2(VGSn – VTn)VDSn – VDSn^2] =

= 8[2*3*Vol – Vol^2] = 8*[6Vol – Vol^2] = 48Vol – 8Vol^2

IR1 = (Vdd – Vout) / (R1 + R2) = 5/45000 – Vol/45000 = 1/9000 – Vol/45000 [A]

IR1 = 1/9 – Vol/45 [mA]

As we already found:

VGS2 = 2*(Vout – Vdd)/3 = Vout*2/3 -10/3 

ISDp = Kp(VSGp + VTp)^2 = 8(10/3 – Vol*2/3 – 0.8)^2 = 8(2.53 – Vol*2/3)^2 =

= 8(6.42 – 3.38Vol + Vol^2*4/9) = 51.34 – 27.02Vol + Vol^2*32/9

Therefore, Eq. I is:

48Vol – 8Vol^2 = 51.34 – 27.02Vol + Vol^2*32/9 + 1/9 – Vol/45

 Vol = 0.779v

Using similar methods, you should find Vm.

Transition points

Voh is also the transition point of M2 from cut to saturation. As we saw, M2 is never linear. Therefore, M2 has no additional transition point.

The transition point of M1 from cut to (linear) conductance is where M1 starts conducting, ie Vin = 0.8v.

Using similar methods to those we used, one should find the transition point of M1 from linear to saturation.

Also, using these methods, one should find the values of both Vin, Vout for each transition point.

שאלה 3

שאלה 4

1 ג.

2 ג.

3 ב. הסבר:

Q6 is cut before Q5, because Q6 has larger bulk effect (due to higher Vsb).

Assuming that the voltage drop across the FETs is small, Vs6>Vs5 => Vsb6>Vsb5 => Vt6>Vt5 => Vdd-Vt6 < Vdd-Vt5.

So, Q6 stop conducting before Q5 does. Q5 can only pass its drain voltage, which is Vdd-Vt6.